Thiết kế DMZ:

Cấu hình cho từng bộ phận:

- Web Server:

Desktop → IP Configuration:

| IP Address | 192.168.80.10 |
| --- | --- |
| Mask | 255.255.255.0 |
| Gateway | 192.168.80.1 |
| DNS | 192.168.80.30 |

Services → HTTP:

| HTTP | ON |
| --- | --- |
| HTTPS | ON |

- DNS Server:

Desktop → IP Configuration:

| IP Address | 192.168.80.30 |
| --- | --- |
| Mask | 255.255.255.0 |
| Gateway | 192.168.80.1 |

Services → DNS

Add Record:

| Name | Address |
| --- | --- |
| www.tmtc.com | 192.168.80.10 |
| mail.tmtc.com | 192.168.80.20 |

- Mail Server:

Desktop → IP Configuration:

| IP Address | 192.168.80.20 |
| --- | --- |
| Mask | 255.255.255.0 |
| Gateway | 192.168.80.1 |

Services → Email

| SMTP | ON |
| --- | --- |
| POP3 | ON |
| Domain | tmtc.com |

- Firewall 1:

Config → Interface:

Fa0/0: 10.10.10.2/30

Fa0/1: 192.168.80.1/24

Static: 200.200.200.0/24 via 10.10.10.1

- Firewall 2:

Config → Interface:

Fa0/0: 192.168.80.254/24

Fa0/1: 172.16.0.1/30

Static: 192.168.0.0/16 via 172.16.0.2, 0.0.0.0/0 via 192.168.80.1

- Router0:

Fa0/0: 10.10.10.1/30

Fa0/1: 200.200.200.1/24

Static: 192.168.80.0/24 via 10.10.10.2

ACL cần làm để đúng mô hình DMZ

Trên Firewall1:

Cho phép Internet truy cập Web:

access-list 100 permit tcp any host 192.168.80.10 eq 80
access-list 100 permit tcp any host 192.168.80.10 eq 443

Cho phép Mail:

access-list 100 permit tcp any host 192.168.80.20 eq 25

Cho phép DNS:

access-list 100 permit udp any host 192.168.80.30 eq 53

Chặn toàn bộ Internet vào LAN:

access-list 100 deny ip any 192.168.0.0 0.0.255.255

Kế hoạch địa chỉ IP:

Do công ty có 100-200 người dùng nên mỗi VLAN sử dụng: /24 (254 host)

Dễ mở rộng.

| VLAN | Phòng ban | Network | Gateway |
| --- | --- | --- | --- |
| 10 | Ban Giám đốc | 192.168.10.0/24 | 192.168.10.1 |
| 20 | Kế toán | 192.168.20.0/24 | 192.168.20.1 |
| 30 | Nhân sự | 192.168.30.0/24 | 192.168.30.1 |
| 40 | Kinh doanh | 192.168.40.0/24 | 192.168.40.1 |
| 50 | IT | 192.168.50.0/24 | 192.168.50.1 |
| 60 | Server Farm | 192.168.60.0/24 | 192.168.60.1 |
| 70 | Guest WiFi | 192.168.70.0/24 | 192.168.70.1 |
| 80 | DMZ | - | 192.168.80.1 |
| 99 | Management | 192.168.99.0/24 | 192.168.99.1 |

Server Farm:

| Thiết bị | IP |
| --- | --- |
| Database Server | 192.168.60.10 |
| File Server | 192.168.60.20 |
| Backup Server | 192.168.60.30 |
| Domain Controller | 192.168.60.40 |

DMZ:

| Thiết bị | IP |
| --- | --- |
| Web Server | 192.168.80.10 |
| Mail Server | 192.168.80.20 |
| DNS Server | 192.168.80.30 |

Phân quyền truy cập:

| Phòng ban | Chính sách bảo mật |
| --- | --- |
| Giám đốc | Truy cập toàn bộ hệ thống |
| Kế toán | Truy cập Database kế toán. Không truy cập dữ liệu nhân sự. |
| Nhân sự | Truy cập hệ thống nhân sự. Không truy cập dữ liệu kế toán. |
| Kinh doanh | Truy cập Website và CRM. Không truy cập Database. |
| IT | Quản trị toàn bộ thiết bị mạng. |
| Server |  |
| Guest WiFi | Chỉ được truy cập Internet. Không được truy cập bất kỳ VLAN nội bộ nào. |
| Management |  |